BASIC PROTOCOL FOR D-SERIES Version 5.25

Introduction

In the previous version (Ver 4.XX) of D-Series, the communication protocol is implemented in a high accuracy packet protocol format but it has disadvantages of complications. In order to make these capabilities as simple as possible, more user friendly, a new and easy protocol has been implemented.  This manual is intended to provide full protocol information for users who want to write their own PC  communication program to incorporate with our signs.

Structure of Protocol :

A very simple protocol structure is illustrated as follow:

<IDXX>...TXT/CMD.. [cr][lf]

Where :

<IDXX> 		Packet header also severed as destination identifier

< & >       are ASCII code 3C & 3D,

ID            are character "I" & "D" must be in Upper case

XX	    are the Hex number 00 to FF in ASCII format i.e.

00		=	display unit 0	(global call)

01		=	display unit 1

0A		=	display unit 10

10		=	 display unit 16

FF		=	display unit 255

...TXT/CMD...	Packet body either Display Data or Command

(refer to Page message & Command for details.)

[cr][lf]			Packet end

[cr]	= ASCII code 0D

[lf]	= ASCII code 0A

Remarks: Each single packet only carries one Display Data or single Command, the followings will be ignored.

All packet(s) must consist of a unique ID number, except SET CLOCK command (see Command section).

An <IDxx> will answer after a successful transfer is made, except SET CLOCK  command and global call.

Display Data:

A)	Page message :

There are 26 pages available in the display sign, the length of each page is dynamic which includes text, graphics and European characters,  their basic format as follow:

<Pn>....MSG.....[cr][lf]

<Pn>		denotes which page this message belongs to :

<,P & > are ASCII character "<" "P" & ">" respectively

n 	is the page number in ASCII character, i.e.

A	=	Page A

B	=	Page B

:		   :

:		   :

Y	=	Page Y

Z	=	Page Z

*Remark: if no <Pn> is specified, page A is assumed as default.

....MSG.....	Contents message data of this page including color informations, character size informations & display functions etc.

Text message:

ASCII characters:

Accept free format text, i.e. any character and symbols( 96 ASCII printable characters 20H - 7FH).

European characters:

72 European characters is provided for multi-nation language applications, they are addressed as follow:

<UA>

<UB>

:

* For the European assignment table, please refer to APPENDIX A

Graphic Blocks:

26 User alterable graphic blocks is provided to enrich the visual effect, they are addressed as follow:

<BA>		Graphic A

<BB>		Graphic B

:

<BY>		Graphic Y

<BZ>		Graphic Z

Color informations:

26 color combinations  selection is allowed.

To define the color of the following character(s) a color attribute indicator  must be placed before it, such as :

<CA>		Low Red

<CB>		Mid Red

<CC>		High Red

:

The default character color is Rainbow <CP>.

Once the attribute indicator is placed then the following character(s)  will be changed to the corresponding color until next attribute is encountered.

* For details please refer to color table in APPENDIX B

Character size:

Different size selection is possible for upto a single character. To define the size of the following character(s),8 combinations of character sizes are available and a  size attribute indicator must be placed before it, such as:

<SA>		Normal size

<SB>		Double size

:

The default character size is Normal <SA>.

Once the attribute indicator is placed then the following characters(s)  will be changed to the corresponding size until next is encountered.

* For details please refer to size table in APPENDIX C

Display functions:

26 unique screen effects & function selections are allowed. To define what is the screen effect of the following character(s) a function indicator  must be placed before it, such as:

<FA>		Auto Function

<FB>

:

The default function is SHIFT LEFT <FS>.

* For details please refer to Function in APPENDIX D

B) Timer (schedule) setting :

There are 10 Timers (schedules) available in the display sign, each of which consists of 32 entries and the basic format is as follow:

<Tn>WHHMMPPP....[cr][lf]

<Tn>	denotes the Timer n, where n = A to J

<,T & > are ASCII character "<" "T" & ">" respectively

n 	is the Timer (schedule) number in ASCII character, i.e.

A	=	Timer A

B	=	Timer B

:

I	=	Timer I

J	=	Timer J

W	denotes the day of week where this schedule activates, the valid values are as follow (in a single ASCII character form):

*	=	Every day of the week

0	=	Sunday

1	=	Monday

:

5	=	Friday

6	=	Saturday

HH	denotes the Hour when this schedule activates, the valid values are as follow (in two ASCII character form, 24 hour notation):

**	=	Every hour of the day

00	=	00 hour mid night

01	=	01 hour mid night

:

13	=	1 o'clock afternoon

:

18	=	6 o'clock evening

:

23	=	11 o'clock night

MM	denotes the minute when this schedule activates, the valid values are as follow (in two ASCII character form):

**		=	Every minute of the hour

00 to 59	=	minutes

PPP....	denotes the Page sequence in this schedule, the length of the sequence is dynamic upto 32 entries, the valid page number ranges from A to Z.

C)	User alterable Graphic Block:

There are 26 (A - Z)  user alterable graphic blocks available,  the basic format is as follow:

<Gn>...CCC...[cr][lf]

<Gn>	<,G& > = ASCII "<","G" & ">"

n= Block graphic number in ONE ASCII character (A - Z)

...CCC...

Bit pattern(with color) information of the graphic block is in fix length of 126 bytes i.e. 18 dots by 7 rows, where upper left dot is the first byte and the lower right is the last (126th) byte. The whole graphic block update will be ignored if any length is longerr than 126.

For each byte it can either be "R" (red), "G" (green), "Y" (yellow) or "B" (black) . Any characters other than "R","G","Y" & "B" are treated as Black.

Example :  Updating graphic block Y with 1 & 2 rows in red, 3,4 & 5 rows in yellow  6 & 7 rows in green.

<ID01><IY>RRRRRRRRRRRRRRRRRRRRRRRRRRRRRRRRRRRRYYYYYYYYYYYYYYYYYYYYYYYYYYYYYYYYYYYYYYYYYYYYYYYYYYYYYGGGGGGGGGGGGGGGGGGGGGGGGGGGGGGGGGGGG[cr][lf]

Commands:

1)	Delete Page(s):

<DPn>[cr][lf]

<,D,P& > = ASCII "<","D","P" & ">"

n  Page number in ASCII character (A - Z & *)

<DP*>	=	Delete All pages

<DPA>	=	Delete page A

:

<DPZ>	=	Delete page Z

2)	Delete Schedule:

<DTn>[cr][lf]

<,D,T& > = ASCII "<","D","T" & ">"

n  Timer (Schedule) number in ASCII character (A - J & *)

<DT*>	=	Delete ALL timers

<DJA>	=	Delete TIMER A

:

<DTJ>	=	Delete TIMER J

3)	Delete Graphic block(s):

<DGn>[cr][lf]

<,D,G& > = ASCII "<","D","G" & ">"

n  Graphic block number in ASCII character (A - Z & *)

<DP*>	=	Delete All graphics

<DPA>	=	Delete Graphic block A

:

<DPZ>	=	Delete Graphic block Z

4)	Delete ALL:

This command will delete all Page(s), Timer(s) and restore all default Graphic 	blocks.

<D*>[cr][lf]

<,D,*& > = ASCII "<","D","*" & ">"

5)	Direct Run Page

<IDXX><RPn>[cr][lf]

<,R,P & > = ASCII “<“,”R”,”P” & “>“

n Page number in ASCII character (A - Z & *)

<RP*>	=	Restart

<RPA> 	=	Run Page A

:

<RPZ>	=	Run Page Z

SET CLOCK command:

This is a special command to sync. or set the relative/real time clock of the display unit(s) and it is the only GLOBAL command in this set of protocol. The format as shown below:

<TYYMMDDWhhmmss>[cr][lf]

<, T & >	ASCII "<", "T" & ">"

YY		Year (00 - 99)

MM		Month (01 - 12)

DD		Day (01 - 31)

W		Day of week (0 - 6)

hh		Hour (in 24 hour format, 00 - 23)

mm		Minute (00 - 59)

ss		Second (00 - 59)

APPENDIX A			European character table

U#	£	UA	Å	UB	Ä	UC	Á	UD	À			UE	Æ	UF	a	UG	Ç	UH	É	UI	È			UJ	ê	UK	Ì	UL	Í	UM	Ñ	UN	Ö

UO	Ó	UP	Ò	UQ	Ø	UR		US	Ü

UT	Ú	UU	Ù	UV		UW	þ	UX	¿

UY	Ð	UZ	Ÿ

U$	¥	Ua	å	Ub	ä	Uc	á	Ud	à

Ue	æ	Uf	â	Ug	ç	Uh	é	Ui	è

Uj	ë	Uk	Ï	Ul	Î	Um	ñ	Un	ö

Uo	ó	Up	ò	Uq	ø	Ur	ô	Us	ü

Ut	ú	Uu	ù	Uv	û	Uw 	ß	Ux	º

Uy	¢	Uz	Ý

APPENDIX B			Color Table for Multi-Color

CA	Dim RED		CJ	Dim LIME		CS	GRN/RED 3D

CB	RED			CK	Bright LIME		CT	GRN/YEL 3D

CC	Bright RED		CL	Bright GREEN		CU	GRN on RED

CD	ORANGE		CM	GREEN		CV	RED on GRN

CE	Bright ORANGE	CN	Dim GREEN		CW	ORG on GRN 3D

CF	LtYELLOW		CO	YEL/GRN/RED		CX	LIME on RED 3D

CG	YELLOW		CP	RAINBOW		CY	GRN on RED 3D

CH	Bright YELLOW	CQ	RED/GRN 3D		CZ	RED on GRN 3D

CI	LIME			CR	RED/YEL 3D

Color Table for  Mono Color

CA	RED

CB	RED REVERSE

APPENDIX C			Size     Table

SA	Normal			SB	Bold			SC	Italic

SD	Bold Italic		SE	Flash Normal		SF	Flash Bold

SG	Flash Italic		SH	Flash Bold Italic

APPENDIX D			Function    Table

FA	AUTO			FB	OPEN <--->		FC	COVER <--->

FD	APPEAR		FE	CYCLING		FF	CLOSE <----

FG	CLOSE ---->		FH	CLOSE --><--		FI	SCROLL UP

FJ	SCROLL DOWN	FK	OVERLAP		FL	STACKING

FM	COMIC 1		FN	COMIC 2		FO	BEEP

FP	PAUSE			FQ	SLEEP			FR	RANDOM

FS	SHIFT <----		FT	TIME/DATE		FU	MAGIC

FV	THANK YOU		FW	WELCOME		FX	SPEED 1

FY	SPEED 2		FZ	SPEED 3

SW1		(BAUD RATE SETTING)

8	7	6	5	4	3	2	1

X	X	X	X	X	0	0	0	2400	baud

X	X	X	X	X	0	0	1	  300	baud

X	X	X	X	X	0	1	0	  600	baud

X	X	X	X	X	0	1	1	1200	baud

X	X	X	X	X	1	0	0	2400	baud

X	X	X	X	X	1	0	1	4800	baud

X	X	X	X	X	1	1	0	9600	baud

X	X	X	X	X	1	1	1	2400	baud

SW2		(BOARD ID SETTING)

8	7	6	5	4	3	2	1

0	0	0	0	0	0	0	0	not allowed (reserve as global)

0	0	0	0	0	0	0	1	ID01

0	0	0	0	0	0	1	0	ID02

:				:

1	1	1	1	1	1	1	0	IDFE	(ID254 decimal)

1	1	1	1	1	1	1	1	IDFF	(ID255 decimal)

DIP switch definiation of Net-02 for D,DX,DZ & DXZ ver 5.25